实验3  加法器和ALU设计

一、实验目的

- 掌握先行进位加法器CLA和先行进位部件CLU的设计方法。

- 掌握32位先行进位加法器及相关标志位的实现方法。

- 掌握ALU的设计方法，根据指令要求实现6种操作的ALU器件。

二、实验环境

Vivado开发工具、ModelSim仿真软件。

三、设计任务与要求

1. 4位先行进位加法器CLA实验

用Verilog HDL实现并验证4位先行进位加法器CLA。

2. 16位两级先行进位加法器实验

用Verilog HDL实现并验证16位两级先行进位加法器。

3. 32位加法器实验

(1) 将两个16位两级先行进位加法器串行级联构建一个32位加法器，并生成CF、SF、OF、ZF标志。

(2) 用Verilog HDL实现并验证上述32位加法器。

4. ALU设计实验

用Verilog HDL实现一个支持9条RV32I指令所包含的6种操作（add、or、slt、sltu、srcB、sub）的32位ALU，并对ALU的功能进行验证。

四、实验内容与步骤

1. 4位先行进位加法器

(a) 工作原理

4位CLA通过每位传播信号P=a^b和产生信号G=a&b展开c1到c4，避免等待逐级串行进位。模块输出sum、cout，同时给出组传播PG和组产生GG，便于构造更高位宽的两级CLA。

(b) Verilog源程序

`timescale 1ns / 1ps

module cla_4bit (
    input  wire [3:0] a,
    input  wire [3:0] b,
    input  wire       cin,
    output wire [3:0] sum,
    output wire       cout,
    output wire       pg,
    output wire       gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c1;
    wire c2;
    wire c3;
    wire c4;

    assign p = a ^ b;
    assign g = a & b;

    assign c1 = g[0] | (p[0] & cin);
    assign c2 = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c3 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c4 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
              | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    assign sum = p ^ {c3, c2, c1, cin};
    assign cout = c4;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module cla_16bit (
    input  wire [15:0] a,
    input  wire [15:0] b,
    input  wire        cin,
    output wire [15:0] sum,
    output wire        cout,
    output wire        pg,
    output wire        gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c4;
    wire c8;
    wire c12;
    wire c16;

    assign c4  = g[0] | (p[0] & cin);
    assign c8  = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c12 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c16 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
               | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    cla_4bit u0 (.a(a[3:0]),    .b(b[3:0]),    .cin(cin), .sum(sum[3:0]),    .cout(), .pg(p[0]), .gg(g[0]));
    cla_4bit u1 (.a(a[7:4]),    .b(b[7:4]),    .cin(c4),  .sum(sum[7:4]),    .cout(), .pg(p[1]), .gg(g[1]));
    cla_4bit u2 (.a(a[11:8]),   .b(b[11:8]),   .cin(c8),  .sum(sum[11:8]),   .cout(), .pg(p[2]), .gg(g[2]));
    cla_4bit u3 (.a(a[15:12]),  .b(b[15:12]),  .cin(c12), .sum(sum[15:12]),  .cout(), .pg(p[3]), .gg(g[3]));

    assign cout = c16;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module adder_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire        sub,
    output wire [31:0] result,
    output wire        CF,
    output wire        SF,
    output wire        OF,
    output wire        ZF
);

    wire [31:0] b_eff;
    wire c16;
    wire cout;
    wire pg0;
    wire gg0;
    wire pg1;
    wire gg1;

    assign b_eff = b ^ {32{sub}};

    cla_16bit u_low (
        .a(a[15:0]),
        .b(b_eff[15:0]),
        .cin(sub),
        .sum(result[15:0]),
        .cout(c16),
        .pg(pg0),
        .gg(gg0)
    );

    cla_16bit u_high (
        .a(a[31:16]),
        .b(b_eff[31:16]),
        .cin(c16),
        .sum(result[31:16]),
        .cout(cout),
        .pg(pg1),
        .gg(gg1)
    );

    assign CF = sub ? ~cout : cout;
    assign SF = result[31];
    assign OF = (a[31] & b_eff[31] & ~result[31]) | (~a[31] & ~b_eff[31] & result[31]);
    assign ZF = (result == 32'b0);

endmodule

module alu_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire [2:0]  alu_ctrl,
    output reg  [31:0] result,
    output reg         CF,
    output wire        SF,
    output reg         OF,
    output wire        ZF
);

    localparam ALU_ADD  = 3'b000;
    localparam ALU_SUB  = 3'b001;
    localparam ALU_OR   = 3'b010;
    localparam ALU_SLT  = 3'b011;
    localparam ALU_SLTU = 3'b100;
    localparam ALU_SRCB = 3'b101;

    wire [31:0] add_result;
    wire add_cf;
    wire add_sf;
    wire add_of;
    wire add_zf;
    wire use_sub;

    assign use_sub = (alu_ctrl == ALU_SUB) || (alu_ctrl == ALU_SLT) || (alu_ctrl == ALU_SLTU);

    adder_32bit u_adder (
        .a(a),
        .b(b),
        .sub(use_sub),
        .result(add_result),
        .CF(add_cf),
        .SF(add_sf),
        .OF(add_of),
        .ZF(add_zf)
    );

    always @(*) begin
        result = 32'b0;
        CF = 1'b0;
        OF = 1'b0;

        case (alu_ctrl)
            ALU_ADD: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_SUB: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_OR: begin
                result = a | b;
            end
            ALU_SLT: begin
                result = {31'b0, (add_sf ^ add_of)};
            end
            ALU_SLTU: begin
                result = {31'b0, add_cf};
            end
            ALU_SRCB: begin
                result = b;
            end
            default: begin
                result = 32'b0;
            end
        endcase
    end

    assign SF = result[31];
    assign ZF = (result == 32'b0);

endmodule

(c) RTL视图

(d) ModelSim源程序

文件：lab3/task1_cla_4bit/sim/tb_cla_4bit.v

`timescale 1ns / 1ps

module tb_cla_4bit;

    reg [3:0] a;
    reg [3:0] b;
    reg cin;
    wire [3:0] sum;
    wire cout;
    wire pg;
    wire gg;

    integer ai;
    integer bi;
    integer ci;
    integer error_count;
    reg [4:0] expected;

    cla_4bit uut (
        .a(a),
        .b(b),
        .cin(cin),
        .sum(sum),
        .cout(cout),
        .pg(pg),
        .gg(gg)
    );

    initial begin
        error_count = 0;
        for (ai = 0; ai < 16; ai = ai + 1) begin
            for (bi = 0; bi < 16; bi = bi + 1) begin
                for (ci = 0; ci < 2; ci = ci + 1) begin
                    a = ai[3:0];
                    b = bi[3:0];
                    cin = ci[0];
                    expected = ai + bi + ci;
                    #1;
                    if ({cout, sum} !== expected) begin
                        $display("ERROR a=%h b=%h cin=%b got=%h expected=%h", a, b, cin, {cout, sum}, expected);
                        error_count = error_count + 1;
                    end
                end
            end
        end

        if (error_count == 0) begin
            $display("========================================");
            $display("Simulation PASSED. No errors found.");
            $display("========================================");
        end else begin
            $display("========================================");
            $display("Simulation FAILED. Error count = %0d", error_count);
            $display("========================================");
        end

        $finish;
    end

endmodule

仿真脚本：lab3/task1_cla_4bit/sim/run.do

transcript on
onbreak {resume}
onerror {quit -code 1}

set sim_dir [file normalize [pwd]]
set root_dir [file normalize [file join $sim_dir ".."]]
set top_module "cla_4bit"
set sim_top "tb_cla_4bit"

cd $sim_dir

if {[file exists work]} {
    vdel -lib work -all
}

vlib work
vmap work work

set src_v_files  [lsort [glob -nocomplain [file join $root_dir "src" "*.v"]]]
set src_sv_files [lsort [glob -nocomplain [file join $root_dir "src" "*.sv"]]]
set tb_v_files   [lsort [glob -nocomplain [file join $sim_dir "*.v"]]]
set tb_sv_files  [lsort [glob -nocomplain [file join $sim_dir "*.sv"]]]
set src_files [concat $src_v_files $src_sv_files]
set tb_files  [concat $tb_v_files $tb_sv_files]

if {[llength $src_files] == 0} {
    puts "ERROR: No RTL source files found in [file join $root_dir src]"
    quit -code 1
}

if {[llength $tb_files] == 0} {
    puts "ERROR: No testbench files found in $sim_dir"
    quit -code 1
}

puts "========================================"
puts "Compile RTL sources"
puts "========================================"
foreach file $src_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Compile simulation sources"
puts "========================================"
foreach file $tb_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Start simulation: work.$sim_top"
puts "========================================"
vsim -voptargs=+acc work.$sim_top
onfinish stop

if {[batch_mode]} {
    log -r /*
} elseif {[file exists [file join $sim_dir "wave.do"]]} {
    do [file join $sim_dir "wave.do"]
} else {
    log -r /*
    add wave -r /*
}

run -all

set error_count_value ""
if {![catch {examine -radix decimal /$sim_top/error_count} error_count_value]} {
    puts "Self-check error_count = $error_count_value"
    if {$error_count_value != 0} {
        puts "ERROR: Self-checking testbench reported failures."
        quit -code 1
    }
} else {
    puts "WARNING: No /$sim_top/error_count signal found; relying on testbench exit status."
}

puts "========================================"
puts "ModelSim flow finished."
puts "Top module: $top_module"
puts "Simulation top: $sim_top"
puts "========================================"

quit -code 0

(e) ModelSim仿真结果

2. 16位两级先行进位加法器

(a) 工作原理

16位两级CLA由4个4位CLA组成，下层产生每4位的PG/GG，上层先行进位逻辑计算4、8、12、16位边界进位。相比串行级联，该结构缩短进位等待链路。

(b) Verilog源程序

`timescale 1ns / 1ps

module cla_4bit (
    input  wire [3:0] a,
    input  wire [3:0] b,
    input  wire       cin,
    output wire [3:0] sum,
    output wire       cout,
    output wire       pg,
    output wire       gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c1;
    wire c2;
    wire c3;
    wire c4;

    assign p = a ^ b;
    assign g = a & b;

    assign c1 = g[0] | (p[0] & cin);
    assign c2 = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c3 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c4 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
              | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    assign sum = p ^ {c3, c2, c1, cin};
    assign cout = c4;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module cla_16bit (
    input  wire [15:0] a,
    input  wire [15:0] b,
    input  wire        cin,
    output wire [15:0] sum,
    output wire        cout,
    output wire        pg,
    output wire        gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c4;
    wire c8;
    wire c12;
    wire c16;

    assign c4  = g[0] | (p[0] & cin);
    assign c8  = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c12 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c16 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
               | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    cla_4bit u0 (.a(a[3:0]),    .b(b[3:0]),    .cin(cin), .sum(sum[3:0]),    .cout(), .pg(p[0]), .gg(g[0]));
    cla_4bit u1 (.a(a[7:4]),    .b(b[7:4]),    .cin(c4),  .sum(sum[7:4]),    .cout(), .pg(p[1]), .gg(g[1]));
    cla_4bit u2 (.a(a[11:8]),   .b(b[11:8]),   .cin(c8),  .sum(sum[11:8]),   .cout(), .pg(p[2]), .gg(g[2]));
    cla_4bit u3 (.a(a[15:12]),  .b(b[15:12]),  .cin(c12), .sum(sum[15:12]),  .cout(), .pg(p[3]), .gg(g[3]));

    assign cout = c16;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module adder_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire        sub,
    output wire [31:0] result,
    output wire        CF,
    output wire        SF,
    output wire        OF,
    output wire        ZF
);

    wire [31:0] b_eff;
    wire c16;
    wire cout;
    wire pg0;
    wire gg0;
    wire pg1;
    wire gg1;

    assign b_eff = b ^ {32{sub}};

    cla_16bit u_low (
        .a(a[15:0]),
        .b(b_eff[15:0]),
        .cin(sub),
        .sum(result[15:0]),
        .cout(c16),
        .pg(pg0),
        .gg(gg0)
    );

    cla_16bit u_high (
        .a(a[31:16]),
        .b(b_eff[31:16]),
        .cin(c16),
        .sum(result[31:16]),
        .cout(cout),
        .pg(pg1),
        .gg(gg1)
    );

    assign CF = sub ? ~cout : cout;
    assign SF = result[31];
    assign OF = (a[31] & b_eff[31] & ~result[31]) | (~a[31] & ~b_eff[31] & result[31]);
    assign ZF = (result == 32'b0);

endmodule

module alu_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire [2:0]  alu_ctrl,
    output reg  [31:0] result,
    output reg         CF,
    output wire        SF,
    output reg         OF,
    output wire        ZF
);

    localparam ALU_ADD  = 3'b000;
    localparam ALU_SUB  = 3'b001;
    localparam ALU_OR   = 3'b010;
    localparam ALU_SLT  = 3'b011;
    localparam ALU_SLTU = 3'b100;
    localparam ALU_SRCB = 3'b101;

    wire [31:0] add_result;
    wire add_cf;
    wire add_sf;
    wire add_of;
    wire add_zf;
    wire use_sub;

    assign use_sub = (alu_ctrl == ALU_SUB) || (alu_ctrl == ALU_SLT) || (alu_ctrl == ALU_SLTU);

    adder_32bit u_adder (
        .a(a),
        .b(b),
        .sub(use_sub),
        .result(add_result),
        .CF(add_cf),
        .SF(add_sf),
        .OF(add_of),
        .ZF(add_zf)
    );

    always @(*) begin
        result = 32'b0;
        CF = 1'b0;
        OF = 1'b0;

        case (alu_ctrl)
            ALU_ADD: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_SUB: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_OR: begin
                result = a | b;
            end
            ALU_SLT: begin
                result = {31'b0, (add_sf ^ add_of)};
            end
            ALU_SLTU: begin
                result = {31'b0, add_cf};
            end
            ALU_SRCB: begin
                result = b;
            end
            default: begin
                result = 32'b0;
            end
        endcase
    end

    assign SF = result[31];
    assign ZF = (result == 32'b0);

endmodule

(c) RTL视图

(d) ModelSim源程序

文件：lab3/task2_cla_16bit/sim/tb_cla_16bit.v

`timescale 1ns / 1ps

module tb_cla_16bit;

    reg [15:0] a;
    reg [15:0] b;
    reg cin;
    wire [15:0] sum;
    wire cout;
    wire pg;
    wire gg;

    integer i;
    integer error_count;
    reg [16:0] expected;

    cla_16bit uut (
        .a(a),
        .b(b),
        .cin(cin),
        .sum(sum),
        .cout(cout),
        .pg(pg),
        .gg(gg)
    );

    task run_case;
        input [15:0] ta;
        input [15:0] tb;
        input tcin;
        begin
            a = ta;
            b = tb;
            cin = tcin;
            expected = ta + tb + tcin;
            #1;
            if ({cout, sum} !== expected) begin
                $display("ERROR a=%h b=%h cin=%b got=%h expected=%h", a, b, cin, {cout, sum}, expected);
                error_count = error_count + 1;
            end else begin
                $display("PASS  a=%h b=%h cin=%b result=%h", a, b, cin, {cout, sum});
            end
        end
    endtask

    initial begin
        error_count = 0;
        run_case(16'h0000, 16'h0000, 1'b0);
        run_case(16'hffff, 16'h0001, 1'b0);
        run_case(16'h1234, 16'h4321, 1'b1);
        run_case(16'h8000, 16'h8000, 1'b0);

        for (i = 0; i < 200; i = i + 1) begin
            run_case($random, $random, i[0]);
        end

        if (error_count == 0) begin
            $display("========================================");
            $display("Simulation PASSED. No errors found.");
            $display("========================================");
        end else begin
            $display("========================================");
            $display("Simulation FAILED. Error count = %0d", error_count);
            $display("========================================");
        end

        $finish;
    end

endmodule

仿真脚本：lab3/task2_cla_16bit/sim/run.do

transcript on
onbreak {resume}
onerror {quit -code 1}

set sim_dir [file normalize [pwd]]
set root_dir [file normalize [file join $sim_dir ".."]]
set top_module "cla_16bit"
set sim_top "tb_cla_16bit"

cd $sim_dir

if {[file exists work]} {
    vdel -lib work -all
}

vlib work
vmap work work

set src_v_files  [lsort [glob -nocomplain [file join $root_dir "src" "*.v"]]]
set src_sv_files [lsort [glob -nocomplain [file join $root_dir "src" "*.sv"]]]
set tb_v_files   [lsort [glob -nocomplain [file join $sim_dir "*.v"]]]
set tb_sv_files  [lsort [glob -nocomplain [file join $sim_dir "*.sv"]]]
set src_files [concat $src_v_files $src_sv_files]
set tb_files  [concat $tb_v_files $tb_sv_files]

if {[llength $src_files] == 0} {
    puts "ERROR: No RTL source files found in [file join $root_dir src]"
    quit -code 1
}

if {[llength $tb_files] == 0} {
    puts "ERROR: No testbench files found in $sim_dir"
    quit -code 1
}

puts "========================================"
puts "Compile RTL sources"
puts "========================================"
foreach file $src_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Compile simulation sources"
puts "========================================"
foreach file $tb_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Start simulation: work.$sim_top"
puts "========================================"
vsim -voptargs=+acc work.$sim_top
onfinish stop

if {[batch_mode]} {
    log -r /*
} elseif {[file exists [file join $sim_dir "wave.do"]]} {
    do [file join $sim_dir "wave.do"]
} else {
    log -r /*
    add wave -r /*
}

run -all

set error_count_value ""
if {![catch {examine -radix decimal /$sim_top/error_count} error_count_value]} {
    puts "Self-check error_count = $error_count_value"
    if {$error_count_value != 0} {
        puts "ERROR: Self-checking testbench reported failures."
        quit -code 1
    }
} else {
    puts "WARNING: No /$sim_top/error_count signal found; relying on testbench exit status."
}

puts "========================================"
puts "ModelSim flow finished."
puts "Top module: $top_module"
puts "Simulation top: $sim_top"
puts "========================================"

quit -code 0

(e) ModelSim仿真结果

3. 32位加法器

(a) 工作原理

32位加/减法器由两个16位CLA级联构成。减法通过B取反并令初始进位为1实现A-B；CF表示无符号加法进位或减法无借位，SF取结果最高位，ZF表示结果为0，OF按有符号加减法的符号关系判断溢出。

(b) Verilog源程序

`timescale 1ns / 1ps

module cla_4bit (
    input  wire [3:0] a,
    input  wire [3:0] b,
    input  wire       cin,
    output wire [3:0] sum,
    output wire       cout,
    output wire       pg,
    output wire       gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c1;
    wire c2;
    wire c3;
    wire c4;

    assign p = a ^ b;
    assign g = a & b;

    assign c1 = g[0] | (p[0] & cin);
    assign c2 = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c3 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c4 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
              | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    assign sum = p ^ {c3, c2, c1, cin};
    assign cout = c4;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module cla_16bit (
    input  wire [15:0] a,
    input  wire [15:0] b,
    input  wire        cin,
    output wire [15:0] sum,
    output wire        cout,
    output wire        pg,
    output wire        gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c4;
    wire c8;
    wire c12;
    wire c16;

    assign c4  = g[0] | (p[0] & cin);
    assign c8  = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c12 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c16 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
               | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    cla_4bit u0 (.a(a[3:0]),    .b(b[3:0]),    .cin(cin), .sum(sum[3:0]),    .cout(), .pg(p[0]), .gg(g[0]));
    cla_4bit u1 (.a(a[7:4]),    .b(b[7:4]),    .cin(c4),  .sum(sum[7:4]),    .cout(), .pg(p[1]), .gg(g[1]));
    cla_4bit u2 (.a(a[11:8]),   .b(b[11:8]),   .cin(c8),  .sum(sum[11:8]),   .cout(), .pg(p[2]), .gg(g[2]));
    cla_4bit u3 (.a(a[15:12]),  .b(b[15:12]),  .cin(c12), .sum(sum[15:12]),  .cout(), .pg(p[3]), .gg(g[3]));

    assign cout = c16;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module adder_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire        sub,
    output wire [31:0] result,
    output wire        CF,
    output wire        SF,
    output wire        OF,
    output wire        ZF
);

    wire [31:0] b_eff;
    wire c16;
    wire cout;
    wire pg0;
    wire gg0;
    wire pg1;
    wire gg1;

    assign b_eff = b ^ {32{sub}};

    cla_16bit u_low (
        .a(a[15:0]),
        .b(b_eff[15:0]),
        .cin(sub),
        .sum(result[15:0]),
        .cout(c16),
        .pg(pg0),
        .gg(gg0)
    );

    cla_16bit u_high (
        .a(a[31:16]),
        .b(b_eff[31:16]),
        .cin(c16),
        .sum(result[31:16]),
        .cout(cout),
        .pg(pg1),
        .gg(gg1)
    );

    assign CF = sub ? ~cout : cout;
    assign SF = result[31];
    assign OF = (a[31] & b_eff[31] & ~result[31]) | (~a[31] & ~b_eff[31] & result[31]);
    assign ZF = (result == 32'b0);

endmodule

module alu_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire [2:0]  alu_ctrl,
    output reg  [31:0] result,
    output reg         CF,
    output wire        SF,
    output reg         OF,
    output wire        ZF
);

    localparam ALU_ADD  = 3'b000;
    localparam ALU_SUB  = 3'b001;
    localparam ALU_OR   = 3'b010;
    localparam ALU_SLT  = 3'b011;
    localparam ALU_SLTU = 3'b100;
    localparam ALU_SRCB = 3'b101;

    wire [31:0] add_result;
    wire add_cf;
    wire add_sf;
    wire add_of;
    wire add_zf;
    wire use_sub;

    assign use_sub = (alu_ctrl == ALU_SUB) || (alu_ctrl == ALU_SLT) || (alu_ctrl == ALU_SLTU);

    adder_32bit u_adder (
        .a(a),
        .b(b),
        .sub(use_sub),
        .result(add_result),
        .CF(add_cf),
        .SF(add_sf),
        .OF(add_of),
        .ZF(add_zf)
    );

    always @(*) begin
        result = 32'b0;
        CF = 1'b0;
        OF = 1'b0;

        case (alu_ctrl)
            ALU_ADD: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_SUB: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_OR: begin
                result = a | b;
            end
            ALU_SLT: begin
                result = {31'b0, (add_sf ^ add_of)};
            end
            ALU_SLTU: begin
                result = {31'b0, add_cf};
            end
            ALU_SRCB: begin
                result = b;
            end
            default: begin
                result = 32'b0;
            end
        endcase
    end

    assign SF = result[31];
    assign ZF = (result == 32'b0);

endmodule

(c) RTL视图

(d) ModelSim源程序

文件：lab3/task3_adder_32bit/sim/tb_adder_32bit.v

`timescale 1ns / 1ps

module tb_adder_32bit;

    reg [31:0] a;
    reg [31:0] b;
    reg sub;
    wire [31:0] result;
    wire CF;
    wire SF;
    wire OF;
    wire ZF;

    integer i;
    integer error_count;
    reg [32:0] unsigned_expected;
    reg [31:0] expected_result;
    reg expected_cf;
    reg expected_of;

    adder_32bit uut (
        .a(a),
        .b(b),
        .sub(sub),
        .result(result),
        .CF(CF),
        .SF(SF),
        .OF(OF),
        .ZF(ZF)
    );

    task run_case;
        input [31:0] ta;
        input [31:0] tb;
        input tsub;
        begin
            a = ta;
            b = tb;
            sub = tsub;
            if (tsub) begin
                unsigned_expected = {1'b0, ta} - {1'b0, tb};
                expected_result = ta - tb;
                expected_cf = (ta < tb);
                expected_of = (ta[31] != tb[31]) && (expected_result[31] != ta[31]);
            end else begin
                unsigned_expected = {1'b0, ta} + {1'b0, tb};
                expected_result = ta + tb;
                expected_cf = unsigned_expected[32];
                expected_of = (ta[31] == tb[31]) && (expected_result[31] != ta[31]);
            end
            #1;
            if (result !== expected_result || CF !== expected_cf || SF !== expected_result[31]
                || OF !== expected_of || ZF !== (expected_result == 32'b0)) begin
                $display("ERROR a=%h b=%h sub=%b result=%h exp=%h CF=%b expCF=%b SF=%b OF=%b expOF=%b ZF=%b",
                         a, b, sub, result, expected_result, CF, expected_cf, SF, OF, expected_of, ZF);
                error_count = error_count + 1;
            end
        end
    endtask

    initial begin
        error_count = 0;
        run_case(32'h00000000, 32'h00000000, 1'b0);
        run_case(32'hffffffff, 32'h00000001, 1'b0);
        run_case(32'h80000000, 32'h80000000, 1'b0);
        run_case(32'h00000000, 32'h00000001, 1'b1);
        run_case(32'h80000000, 32'h00000001, 1'b1);
        run_case(32'h7fffffff, 32'hffffffff, 1'b1);

        for (i = 0; i < 200; i = i + 1) begin
            run_case($random, $random, i[0]);
        end

        if (error_count == 0) begin
            $display("========================================");
            $display("Simulation PASSED. No errors found.");
            $display("========================================");
        end else begin
            $display("========================================");
            $display("Simulation FAILED. Error count = %0d", error_count);
            $display("========================================");
        end

        $finish;
    end

endmodule

仿真脚本：lab3/task3_adder_32bit/sim/run.do

transcript on
onbreak {resume}
onerror {quit -code 1}

set sim_dir [file normalize [pwd]]
set root_dir [file normalize [file join $sim_dir ".."]]
set top_module "adder_32bit"
set sim_top "tb_adder_32bit"

cd $sim_dir

if {[file exists work]} {
    vdel -lib work -all
}

vlib work
vmap work work

set src_v_files  [lsort [glob -nocomplain [file join $root_dir "src" "*.v"]]]
set src_sv_files [lsort [glob -nocomplain [file join $root_dir "src" "*.sv"]]]
set tb_v_files   [lsort [glob -nocomplain [file join $sim_dir "*.v"]]]
set tb_sv_files  [lsort [glob -nocomplain [file join $sim_dir "*.sv"]]]
set src_files [concat $src_v_files $src_sv_files]
set tb_files  [concat $tb_v_files $tb_sv_files]

if {[llength $src_files] == 0} {
    puts "ERROR: No RTL source files found in [file join $root_dir src]"
    quit -code 1
}

if {[llength $tb_files] == 0} {
    puts "ERROR: No testbench files found in $sim_dir"
    quit -code 1
}

puts "========================================"
puts "Compile RTL sources"
puts "========================================"
foreach file $src_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Compile simulation sources"
puts "========================================"
foreach file $tb_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Start simulation: work.$sim_top"
puts "========================================"
vsim -voptargs=+acc work.$sim_top
onfinish stop

if {[batch_mode]} {
    log -r /*
} elseif {[file exists [file join $sim_dir "wave.do"]]} {
    do [file join $sim_dir "wave.do"]
} else {
    log -r /*
    add wave -r /*
}

run -all

set error_count_value ""
if {![catch {examine -radix decimal /$sim_top/error_count} error_count_value]} {
    puts "Self-check error_count = $error_count_value"
    if {$error_count_value != 0} {
        puts "ERROR: Self-checking testbench reported failures."
        quit -code 1
    }
} else {
    puts "WARNING: No /$sim_top/error_count signal found; relying on testbench exit status."
}

puts "========================================"
puts "ModelSim flow finished."
puts "Top module: $top_module"
puts "Simulation top: $sim_top"
puts "========================================"

quit -code 0

(e) ModelSim仿真结果

4. ALU设计

(a) 工作原理

32位ALU复用32位加/减法器实现add和sub，并在控制码选择下输出or、slt、sltu和srcB等结果。slt使用有符号比较，sltu使用无符号比较；SF/ZF由最终结果派生，用于后续CPU数据通路。

(b) Verilog源程序

`timescale 1ns / 1ps

module cla_4bit (
    input  wire [3:0] a,
    input  wire [3:0] b,
    input  wire       cin,
    output wire [3:0] sum,
    output wire       cout,
    output wire       pg,
    output wire       gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c1;
    wire c2;
    wire c3;
    wire c4;

    assign p = a ^ b;
    assign g = a & b;

    assign c1 = g[0] | (p[0] & cin);
    assign c2 = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c3 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c4 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
              | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    assign sum = p ^ {c3, c2, c1, cin};
    assign cout = c4;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module cla_16bit (
    input  wire [15:0] a,
    input  wire [15:0] b,
    input  wire        cin,
    output wire [15:0] sum,
    output wire        cout,
    output wire        pg,
    output wire        gg
);

    wire [3:0] p;
    wire [3:0] g;
    wire c4;
    wire c8;
    wire c12;
    wire c16;

    assign c4  = g[0] | (p[0] & cin);
    assign c8  = g[1] | (p[1] & g[0]) | (p[1] & p[0] & cin);
    assign c12 = g[2] | (p[2] & g[1]) | (p[2] & p[1] & g[0]) | (p[2] & p[1] & p[0] & cin);
    assign c16 = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1])
               | (p[3] & p[2] & p[1] & g[0]) | (p[3] & p[2] & p[1] & p[0] & cin);

    cla_4bit u0 (.a(a[3:0]),    .b(b[3:0]),    .cin(cin), .sum(sum[3:0]),    .cout(), .pg(p[0]), .gg(g[0]));
    cla_4bit u1 (.a(a[7:4]),    .b(b[7:4]),    .cin(c4),  .sum(sum[7:4]),    .cout(), .pg(p[1]), .gg(g[1]));
    cla_4bit u2 (.a(a[11:8]),   .b(b[11:8]),   .cin(c8),  .sum(sum[11:8]),   .cout(), .pg(p[2]), .gg(g[2]));
    cla_4bit u3 (.a(a[15:12]),  .b(b[15:12]),  .cin(c12), .sum(sum[15:12]),  .cout(), .pg(p[3]), .gg(g[3]));

    assign cout = c16;
    assign pg = &p;
    assign gg = g[3] | (p[3] & g[2]) | (p[3] & p[2] & g[1]) | (p[3] & p[2] & p[1] & g[0]);

endmodule

module adder_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire        sub,
    output wire [31:0] result,
    output wire        CF,
    output wire        SF,
    output wire        OF,
    output wire        ZF
);

    wire [31:0] b_eff;
    wire c16;
    wire cout;
    wire pg0;
    wire gg0;
    wire pg1;
    wire gg1;

    assign b_eff = b ^ {32{sub}};

    cla_16bit u_low (
        .a(a[15:0]),
        .b(b_eff[15:0]),
        .cin(sub),
        .sum(result[15:0]),
        .cout(c16),
        .pg(pg0),
        .gg(gg0)
    );

    cla_16bit u_high (
        .a(a[31:16]),
        .b(b_eff[31:16]),
        .cin(c16),
        .sum(result[31:16]),
        .cout(cout),
        .pg(pg1),
        .gg(gg1)
    );

    assign CF = sub ? ~cout : cout;
    assign SF = result[31];
    assign OF = (a[31] & b_eff[31] & ~result[31]) | (~a[31] & ~b_eff[31] & result[31]);
    assign ZF = (result == 32'b0);

endmodule

module alu_32bit (
    input  wire [31:0] a,
    input  wire [31:0] b,
    input  wire [2:0]  alu_ctrl,
    output reg  [31:0] result,
    output reg         CF,
    output wire        SF,
    output reg         OF,
    output wire        ZF
);

    localparam ALU_ADD  = 3'b000;
    localparam ALU_SUB  = 3'b001;
    localparam ALU_OR   = 3'b010;
    localparam ALU_SLT  = 3'b011;
    localparam ALU_SLTU = 3'b100;
    localparam ALU_SRCB = 3'b101;

    wire [31:0] add_result;
    wire add_cf;
    wire add_sf;
    wire add_of;
    wire add_zf;
    wire use_sub;

    assign use_sub = (alu_ctrl == ALU_SUB) || (alu_ctrl == ALU_SLT) || (alu_ctrl == ALU_SLTU);

    adder_32bit u_adder (
        .a(a),
        .b(b),
        .sub(use_sub),
        .result(add_result),
        .CF(add_cf),
        .SF(add_sf),
        .OF(add_of),
        .ZF(add_zf)
    );

    always @(*) begin
        result = 32'b0;
        CF = 1'b0;
        OF = 1'b0;

        case (alu_ctrl)
            ALU_ADD: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_SUB: begin
                result = add_result;
                CF = add_cf;
                OF = add_of;
            end
            ALU_OR: begin
                result = a | b;
            end
            ALU_SLT: begin
                result = {31'b0, (add_sf ^ add_of)};
            end
            ALU_SLTU: begin
                result = {31'b0, add_cf};
            end
            ALU_SRCB: begin
                result = b;
            end
            default: begin
                result = 32'b0;
            end
        endcase
    end

    assign SF = result[31];
    assign ZF = (result == 32'b0);

endmodule

(c) RTL视图

(d) ModelSim源程序

文件：lab3/task4_alu_32bit/sim/tb_alu_32bit.v

`timescale 1ns / 1ps

module tb_alu_32bit;

    localparam ALU_ADD  = 3'b000;
    localparam ALU_SUB  = 3'b001;
    localparam ALU_OR   = 3'b010;
    localparam ALU_SLT  = 3'b011;
    localparam ALU_SLTU = 3'b100;
    localparam ALU_SRCB = 3'b101;

    reg [31:0] a;
    reg [31:0] b;
    reg [2:0] alu_ctrl;
    wire [31:0] result;
    wire CF;
    wire SF;
    wire OF;
    wire ZF;

    integer error_count;

    alu_32bit uut (
        .a(a),
        .b(b),
        .alu_ctrl(alu_ctrl),
        .result(result),
        .CF(CF),
        .SF(SF),
        .OF(OF),
        .ZF(ZF)
    );

    task check_result;
        input [31:0] exp;
        begin
            #1;
            if (result !== exp || SF !== exp[31] || ZF !== (exp == 32'b0)) begin
                $display("ERROR ctrl=%b a=%h b=%h result=%h expected=%h SF=%b ZF=%b",
                         alu_ctrl, a, b, result, exp, SF, ZF);
                error_count = error_count + 1;
            end else begin
                $display("PASS  ctrl=%b a=%h b=%h result=%h", alu_ctrl, a, b, result);
            end
        end
    endtask

    initial begin
        error_count = 0;

        a = 32'h00000005; b = 32'h00000003; alu_ctrl = ALU_ADD;  check_result(32'h00000008);
        a = 32'h00000005; b = 32'h00000007; alu_ctrl = ALU_SUB;  check_result(32'hfffffffe);
        a = 32'h0000000a; b = 32'h00000005; alu_ctrl = ALU_OR;   check_result(32'h0000000f);
        a = 32'hffffffff; b = 32'h00000001; alu_ctrl = ALU_SLT;  check_result(32'h00000001);
        a = 32'hffffffff; b = 32'h00000001; alu_ctrl = ALU_SLTU; check_result(32'h00000000);
        a = 32'h12345678; b = 32'h87654321; alu_ctrl = ALU_SRCB; check_result(32'h87654321);
        a = 32'h80000000; b = 32'h7fffffff; alu_ctrl = ALU_SLT;  check_result(32'h00000001);
        a = 32'h80000000; b = 32'h7fffffff; alu_ctrl = ALU_SLTU; check_result(32'h00000000);

        if (error_count == 0) begin
            $display("========================================");
            $display("Simulation PASSED. No errors found.");
            $display("========================================");
        end else begin
            $display("========================================");
            $display("Simulation FAILED. Error count = %0d", error_count);
            $display("========================================");
        end

        $finish;
    end

endmodule

仿真脚本：lab3/task4_alu_32bit/sim/run.do

transcript on
onbreak {resume}
onerror {quit -code 1}

set sim_dir [file normalize [pwd]]
set root_dir [file normalize [file join $sim_dir ".."]]
set top_module "alu_32bit"
set sim_top "tb_alu_32bit"

cd $sim_dir

if {[file exists work]} {
    vdel -lib work -all
}

vlib work
vmap work work

set src_v_files  [lsort [glob -nocomplain [file join $root_dir "src" "*.v"]]]
set src_sv_files [lsort [glob -nocomplain [file join $root_dir "src" "*.sv"]]]
set tb_v_files   [lsort [glob -nocomplain [file join $sim_dir "*.v"]]]
set tb_sv_files  [lsort [glob -nocomplain [file join $sim_dir "*.sv"]]]
set src_files [concat $src_v_files $src_sv_files]
set tb_files  [concat $tb_v_files $tb_sv_files]

if {[llength $src_files] == 0} {
    puts "ERROR: No RTL source files found in [file join $root_dir src]"
    quit -code 1
}

if {[llength $tb_files] == 0} {
    puts "ERROR: No testbench files found in $sim_dir"
    quit -code 1
}

puts "========================================"
puts "Compile RTL sources"
puts "========================================"
foreach file $src_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Compile simulation sources"
puts "========================================"
foreach file $tb_files {
    puts "vlog $file"
    vlog -work work $file
}

puts "========================================"
puts "Start simulation: work.$sim_top"
puts "========================================"
vsim -voptargs=+acc work.$sim_top
onfinish stop

if {[batch_mode]} {
    log -r /*
} elseif {[file exists [file join $sim_dir "wave.do"]]} {
    do [file join $sim_dir "wave.do"]
} else {
    log -r /*
    add wave -r /*
}

run -all

set error_count_value ""
if {![catch {examine -radix decimal /$sim_top/error_count} error_count_value]} {
    puts "Self-check error_count = $error_count_value"
    if {$error_count_value != 0} {
        puts "ERROR: Self-checking testbench reported failures."
        quit -code 1
    }
} else {
    puts "WARNING: No /$sim_top/error_count signal found; relying on testbench exit status."
}

puts "========================================"
puts "ModelSim flow finished."
puts "Top module: $top_module"
puts "Simulation top: $sim_top"
puts "========================================"

quit -code 0

(e) ModelSim仿真结果

五、实验过程中出现的故障现象、原因分析及解决的办法

CLA设计中P/G定义和进位展开式必须保持一致。解决办法是让4位CLA同时输出PG/GG，并在testbench中除sum/cout外也检查组信号。

32位减法的CF含义容易与借位混淆。实现和报告中统一说明为减法时CF表示无借位条件，验证样例覆盖借位与无借位两类。

ALU的slt与sltu边界不同，使用相同输入分别检查有符号和无符号比较结果，避免把两者混成同一种比较。